Lab 1 & Lab 2 Summary (26/01 - 27/01)

New Proposed Topology
- Replace L3 switches into routers

PT Implementation:

Hardware in PT:
Routers: 2911
Switch: 2960 IOS15
Server: Server-PT

Subnets & IP Address table:

|  | Device | Interface | Connected To | IPv6 Address | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| 0 | Internet Server | Fa0 | Core Router | 2024:3130:2828::2/64 | 2024:3130:2828::1 |
| 1 | Core Router | G0/0 | Internet Server | 2024:3130:2828::1/64 |  |
| 2 |  | G0/1 | Internet Router | 2024:3130:2828:8::1/64 |  |
| 3 | Internet Router | G0/0 | Core Router | 2024:3130:2828:8::2/64 |  |
| 4 |  | G0/1 | Router1 | 2024:3130:2828:7::1/64 |  |
| 5 | Router1 | G0/0 | Internet Router | 2024:3130:2828:7::2/64 |  |
| 6 |  | G0/1 | Lab1 Switch | 2024:3130:2828:1::1/64 |  |
| 7 |  | G0/2 | Lab2 Switch | 2024:3130:2828:2::1/64 |  |
| 9 | Lab1 Switch | G0/1 | Router1 |  |  |
| 10 |  | G0/2 | Lab1 Admin PC |  |  |
| 11 |  | Fa0/1 | Lab1 PC 1 |  |  |
| 12 | Lab1 PCs |  | Lab1 Switch | IPv6 Autoconfig (:1::) | - |
| 13 | Lab2 Switch | G0/1 | Router 1 |  |  |
| 14 |  | G0/2 | Lab2 Admin PC |  |  |
|  |  | Fa0/1 | Lab2 PC 1 |  |  |
|  | Lab2 PCs |  | Lab2 Switch | IPv6 Autoconfig (:2::) | - |

Static Routing:

| At Device: | Outgoing | Incoming |
| --- | --- | --- |
| Router1 | S ::/0 [1/0] via 2024:3130:2828:7::1 Internet Router | . |
| Internet Router | S ::/0 [1/0] via 2024:3130:2828:8::1 Core Router | S 2024:3130:2828:1::/64 [1/0] via 2024:3130:2828:7::2 S 2024:3130:2828:2::/64 [1/0] via 2024:3130:2828:7::2 Router 1 Subnet 1 & 2 |
| Core Router |  | S 2024:3130:2828:1::/64 [1/0] via 2024:3130:2828:8::2 S 2024:3130:2828:2::/64 [1/0] via 2024:3130:2828:8::2 Internet Router Router 1 Subnet 1 & 2 |

Configurations:

| Router1 | enable conf t hostname Router-1 ipv6 unicast-routing --- interface gig0/0 description “Link to Internet Router” ipv6 address 2024:3130:2828:7::2/64 no shutdown interface gig0/1 description “Link to Lab1-Switch” ipv6 address 2024:3130:2828:1::1/64 no shutdown interface gig0/2 description “Link to Lab2-Switch” ipv6 address 2024:3130:2828:2::1/64 no shutdown --- ipv6 route ::/0 2024:3130:2828:7::1 |
| --- | --- |
| Internet Router | enable conf t hostname Internet-Router ipv6 unicast-routing --- interface gig0/0 description “Link to Core Router” ipv6 address 2024:3130:2828:8::2/64 no shutdown interface gig0/1 description “Link to Router1” ipv6 address 2024:3130:2828:7::1/64 no shutdown --- ipv6 route ::/0 2024:3130:2828:8::1 ipv6 route 2024:3130:2828:1::/64 2024:3130:2828:7::2 ipv6 route 2024:3130:2828:2::/642024:3130:2828:7::2 |
| Core Router | enable conf t hostname Core-Router ipv6 unicast-routing --- interface gig0/0 description “Link to Internet Server” ipv6 address 2024:3130:2828::1/64 no shutdown interface gig0/1 description “Link to Internet Router” ipv6 address 2024:3130:2828:8::1/64 no shutdown --- 2024:3130:2828:1::/64 2024:3130:2828:8::2 2024:3130:2828:2::/64 2024:3130:2828:8::2 |

PC IP Configurations:

Automatic

Internet Server Services Configuration:

HTTP, HTTPS enabled

DNS enabled
- Add: www.example.com; 23.1.2.2

Servers (28/01)

Subnets & IP Address table

|  | Device | Interface | Connected To | IPv6 Address | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |
| 1 | Core Router | G0/2 | Server-Router | 2024:3130:2828:9::1/64 |  |
| 2 | Server Router | G0/0 | Core-Router | 2024:3130:2828:9::2/64 |  |
| 3 |  | G0/1 | AdminTutor-Switch | 2024:3130:2828:5::1/64 |  |
| 4 |  | G0/2 | Server-Switch | 2024:3130:2828:6::1/64 |  |
| 5 | AdminTutor Switch | Fa0/1 | Router 1 |  |  |
| 6 |  | G0/1 | Administrator-PC |  |  |
| 7 |  | G0/2 | Tutor-PC |  |  |
| 8 | Administrator PC |  | AdminTutor-Switch | IPv6 Autoconfig (:5::) Make sure DHCP pool and other config flag is enabled |  |
| 9 | Tutor PC |  |  |  |  |
| 10 | Server Switch | Fa0/1 | Router1 |  |  |
| 11 |  | Fa0/2 | DNS-Server |  |  |
| 12 |  | G0/1 | Web-Server |  |  |
| 13 |  | G0/2 | File-Server |  |  |
| 14 | DNS Server |  | Server-Switch | 2024:3130:2828:6::12/64 | 2024:3130:2828:6::1 |
| 15 | Web Server |  |  | 2024:3130:2828:6::10/64 | 2024:3130:2828:6::1 |
| 16 | File Server |  |  | 2024:3130:2828:6::11/64 | 2024:3130:2828:6::1 |

Configurations:

| Core Router | interface gig0/2 description “Link to Server Router” ipv6 address 2024:3130:2828:9::1/64 no shutdown |
| --- | --- |
| Server Router | enable conf t hostname Server-Router ipv6 unicast-routing --- interface gig0/0 description “Link to Core Router” ipv6 address 2024:3130:2828:9::2/64 no shutdown --- interface gig0/1 description “Link to Tutor-Switch” ipv6 address 2024:3130:2828:5::1/64 no shutdown ipv6 dhcp pool DNSPOOL dns-server 2024:3130:2828:6::12 ipv6 dhcp server DNSPOOL ipv6 nd other-config-flag --- interface gig0/2 description “Link to Server-Switch” ipv6 address 2024:3130:2828:6::1/64 no shutdown |

Server Configurations

| Server | Config |
| --- | --- |
| Web-Server | IPv6 Address [2024:3130:2828:6::10] Default Gateway [2024:3130:2828:6::1] |
|  | HTTP & HTTPS [ON], Edit index.html |
| File-Server | IPv6 Address [2024:3130:2828:6::11] Default Gateway [2024:3130:2828:6::1] |
|  | FTP [ON] |
| DNS-Server | IPv6 Address [2024:3130:2828:6::12] Default Gateway [2024:3130:2828:6::1] |
|  | DNS [ON] [www.newitcompany.com, 2024:3130:2828:6::10, AAAA] [files.newitcompany.com, 2024:3130:2828:6::11, AAAA] |

REFERENCE / NOTES

Below is a step-by-step guide for configuring the provided network topology in Cisco Packet Tracer.

Step 1: Core Router Configuration

Core Router is the central hub connecting the other routers.

- Access the Core Router CLI:

- enable

- configure terminal

- hostname CoreRouter

- ipv6 unicast-routing

- Configure the interfaces:

- interface gigabitEthernet 0/0

- description Link to Internet Router

- ipv6 address 2024:3130:2828:7::1/64

- no shutdown

- interface gigabitEthernet 0/1

- description Link to Server Router

- ipv6 address 2024:3130:2828:7::2/64

- no shutdown

Step 2: Internet Router Configuration

Internet Router connects to the internet and other internal routers.

- Access the CLI and configure:

- enable

- configure terminal

- hostname InternetRouter

- ipv6 unicast-routing

- Configure the interfaces:

- interface gigabitEthernet 0/0

- description Link to Core Router

- ipv6 address 2024:3130:2828:7::3/64

- no shutdown

- interface gigabitEthernet 0/1

- description Link to Router 1

- ipv6 address 2024:3130:2828:8::1/64

- no shutdown

- interface gigabitEthernet 0/2

- description Link to Router 2

- ipv6 address 2024:3130:2828:9::1/64

- no shutdown

Step 3: Router 1 Configuration

Router 1 connects to Lab 1 and Lab 2.

- Access the CLI:

- enable

- configure terminal

- hostname Router1

- ipv6 unicast-routing

- Configure the interfaces:

- interface gigabitEthernet 0/0

- description Link to Internet Router

- ipv6 address 2024:3130:2828:8::2/64

- no shutdown

- interface gigabitEthernet 0/1

- description Link to Lab 1

- ipv6 address 2024:3130:2828:1::1/64

- no shutdown

- interface gigabitEthernet 0/2

- description Link to Lab 2

- ipv6 address 2024:3130:2828:2::1/64

- no shutdown

Step 4: Router 2 Configuration

Router 2 connects to the Classroom and SysAdmin segments.

- Access the CLI:

- enable

- configure terminal

- hostname Router2

- ipv6 unicast-routing

- Configure the interfaces:

- interface gigabitEthernet 0/0

- description Link to Internet Router

- ipv6 address 2024:3130:2828:9::2/64

- no shutdown

- interface gigabitEthernet 0/1

- description Link to Classroom

- ipv6 address 2024:3130:2828:3::1/64

- no shutdown

- interface gigabitEthernet 0/2

- description Link to SysAdmin

- ipv6 address 2024:3130:2828:4::1/64

- no shutdown

Step 5: Server Router Configuration

Server Router connects to Administration and Tutor PCs, as well as the Web/File Servers.

- Access the CLI:

- enable

- configure terminal

- hostname ServerRouter

- ipv6 unicast-routing

- Configure the interfaces:

- interface gigabitEthernet 0/0

- description Link to Core Router

- ipv6 address 2024:3130:2828:7::4/64

- no shutdown

- interface gigabitEthernet 0/1

- description Link to Administration and Tutor PCs

- ipv6 address 2024:3130:2828:5::1/64

- no shutdown

- interface gigabitEthernet 0/2

- description Link to Web and File Servers

- ipv6 address 2024:3130:2828:6::1/64

- no shutdown

Step 6: Switch Configuration

Each switch is Layer 2, so minimal configuration is required.

- For Lab 1 Switch (2960):

- enable

- configure terminal

- vlan 1

- name Lab1

- interface vlan 1

- ipv6 address 2024:3130:2828:1::2/64

- no shutdown

Repeat similar steps for each switch, updating the VLAN name and IP address to match the corresponding subnet.

Step 7: End Device Configuration

Configure each end device (PCs/Servers) with an IPv6 address and default gateway:

- IPv6 Address: From the respective subnet (e.g., 2024:3130:2828:1::X/64).

- Default Gateway: The router interface IPv6 address (e.g., 2024:3130:2828:1::1).

Step 8: Testing Connectivity

- Use ping to test IPv6 connectivity between devices.

- Use traceroute for route verification:

- traceroute ipv6 2024:3130:2828:6::2

This step-by-step setup ensures a properly functioning network in Cisco Packet Tracer. Let me know if you need further assistance!

I have created the addressing table for your network and displayed it. Let me know if you need any modifications or further details.

|  | Device | Interface | Connected To | IPv6 Address |
| --- | --- | --- | --- | --- |
| 1 | Core Router | G0/0 | Internet Router | 2024:3130:2828:7::1/64 |
| 2 | Internet Router | G0/1 | Server Router | 2024:3130:2828:7::2/64 |
| 3 | Router 1 | G0/0 | Core Router | 2024:3130:2828:8::1/64 |
| 4 | Router 1 | G0/1 | Lab 1 Switch | 2024:3130:2828:1::1/64 |
| 5 | Router 2 | G0/2 | Lab 2 Switch | 2024:3130:2828:2::1/64 |
| 6 | Router 2 | G0/0 | Internet Router | 2024:3130:2828:9::1/64 |
| 7 | Server Router | G0/1 | Classroom Switch | 2024:3130:2828:3::1/64 |
| 8 | Server Router | G0/2 | SysAdmin Switch | 2024:3130:2828:4::1/64 |
| 9 | Lab 1 Switch | G0/1 | Router 1 | 2024:3130:2828:1::2/64 |
| 10 | Lab 2 Switch | G0/2 | Router 1 | 2024:3130:2828:2::2/64 |
| 11 | Classroom Switch | G0/1 | Router 2 | 2024:3130:2828:3::2/64 |
| 12 | SysAdmin Switch | G0/2 | Router 2 | 2024:3130:2828:4::2/64 |
| 13 | Admin & Tutor Switch | G0/1 | Server Router | 2024:3130:2828:5::2/64 |
| 14 | Web & File Server Switch | G0/2 | Server Router | 2024:3130:2828:6::2/64 |

Static Routing:

At Router 1:

S   ::/0 [1/0]

via 2024:3130:2828:7::1

S   2024:2120:2828::/64 [1/0]

via 2024:3130:2828:1::1

via 2024:3130:2828:2::1

At Internet Router:
ipv6 route 2024:3130:2828:1::/64 2024:3130:2828:7::2	
- Handles routing towards to subnet 1

ipv6 route 2024:3130:2828:2::/64 2024:3130:2828:7::2
- Handles routing towards to subnet 2

S   ::/0 [1/0]

via 2024:3130:2828:8::1

S   2024:3130:2828:1::/64 [1/0]

via 2024:3130:2828:7::2

S   2024:3130:2828:2::/64 [1/0]

via 2024:3130:2828:7::2

At Router Core:

S   2024:3130:2828:1::/64 [1/0]

via 2024:3130:2828:8::2
S   2024:3130:2828:2::/64 [1/0]

via 2024:3130:2828:8::2